2.1.4.3 Bluetooth Transmission

A custom BLE service is used instead of a UART. The custom interface is easier to use, and has simpler troubleshooting capabilities. A UART BLE service has the added layer of simulating a real UART. A custom BLE service with Good and Bad step characteristics, allowing the use of Bluetooth’s built-in notification support. This makes near real-time updates of good and bad steps possible as the Bangle.js smartwatch gets notified immediately when a good or bad step occurs.

# 2.1.4.2 Haptic Vibrator

# 2.1.4.2 Haptic Vibrator

It was discovered that the digital pins on the XIAO Seeed board were unable to supply sufficient current to operate the feedback motor, so the design was switched to use a PNP transistor with a pull-up resistor controlled by a digital pin. The pull-up resistor used was 24k Ohms, but any arbitrarily large resistance would suffice.

### 2.1.2 Sensor Input

Previous design called for five sensor arrays. Current Design utilizes four sensor arrays.

##### 2.1.3.2.3 GI Communication

Trigger voltage for registering a step is now ~ 0.5V.

3.0 Device Overview

## 3.1 hardware modules

The project does not use TPE and only uses TPU for our 3D printed insoles.

### 3.2.3 Prusa Slicer 2.6.1 & Fusion 360

Prusa Version 2.7 is used. The printers, programs, and files are updated.